MAUD Format description

WK Aug 2012

Extension:

ASC

Header:  (11 lines + comment lines)

Hash (#)

followed by key_1 (C,A,S,P,L)

optional: followed by key_2 (OMEGA, CHI, PHI, ETA, - Group)

followed by entries in free format

comment lines, starting with # can be inserted in the header

#C  Ferritic parent:  green-on-top; red-facing-beam 		Title: string

#C  Instrument: GEM						instrument: string

#C  GEM59673						run number: string

#C  User Artioli						user: string

#A  OMEGA 90.00						Angle: default 90.0 :  real

#A  CHI           0.00						Angle: default    0.0 : real

#A  PHI           0.00						Angle: default    0.0 : real

#A  ETA          0.00						Angle: default    0.0 : real

#S    1 - Group  4						S: Bank number ,  Group number

#P0 5.91900  0.0008   9.70508  19.3692			P0: default: 0.0  0.0  0.0  0.0 :  4 real

#L d-Spacing Data Error					L: delimiter between data sets: string

intermediate Headers: (between data sets)  (3 lines)

#S    155 - Group  158						S: Bank number  ,  Group number

#P0 5.91900  0.0008   9.70508  19.3692			P0: default: 0.0  0.0  0.0  0.0 :  4 real

#L d-Spacing Data Error					L: delimiter between data sets: string

Data: 3 columns, free format

D-spacing  Count  Error

Example:

#C  Ferritic parent:  green-on-top; red-facing-beam 		Title: string

#C  Instrument: GEM						instrument: string

#C  GEM59673						run number: string

#C  User Artioli						user: string

#A  OMEGA 90.00						Angle: default 90.0 :  real

#A  CHI           0.00						Angle: default    0.0 : real

#A  PHI           0.00						Angle: default    0.0 : real

#A  ETA          0.00						Angle: default    0.0 : real

#S    1 - Group  4						S: Bank number ,  Group number

#P0 5.91900  0.0008   9.70508  19.3692			P0: default: 0.0  0.0  0.0  0.0 :  4 real

#L d-Spacing Data Error					L: delimiter between data sets: string

0.57474        0.26883341        0.32278325

0.57544        0.34688612        0.34782602

0.57614        0.01801507        0.06690679

0.57684        0.15547407        0.20095458

.

.

.

37.50219        0.44969442        0.85646189

37.54789        1.01267478        1.28501060

37.59364        0.13502131        0.47191016

37.63945        0.00000000        0.00000000

37.67399        0.00000000        0.00000000

#S    2 - Group   5						S-card: Bank # and Group #

#P0 0.0 0.0 0.0 0.0						P0: default: 0.0  0.0  0.0  0.0 :  4 real

#L d-Spacing Data Error					L-card: delimiter between data sets

0.55954        0.37017071        0.36855757

0.56019        0.72134597        0.49122789

0.56084        0.87881955        0.60888285

.

.

.